Design and Hardware Implementation of a Five-Stage Pipelined RV32I RISC-V Processor

Objective

Educational RTL implementation. Interview portfolio project. Synthesizable on FPGA. Clean modular RTL. Emphasis on readability over aggressive optimization.

Architectural Constraints

RV32I subset. Classical 5-stage pipeline. One instruction retired per cycle (ideal case). No speculative execution. No caches. No out-of-order execution.

Design Principles

Modular RTL with one hardware block per module. Synchronous design using a single global clock.

Separation of data path and control logic. Stage-local combinational logic separated by pipeline registers.

Approach

philosophy → scope → architecture → signals → registers → data path details → verification

Documentation

The project documentation work is divided into 14 chapters:

Chapter 1. Project Overview

Chapter 2. Overall Architecture

Chapter 3. Supported Instruction Set

Chapter 4. Datapath

Chapter 5. Pipeline Registers

Chapter 6. Control Signals

Chapter 7. ALU Specification

Chapter 8. Immediate Generator

Chapter 9. Branch and Control-Flow Unit

Chapter 10. Hazard Handling

Chapter 11. Memory System

Chapter 12. Register File

Chapter 13. Module List

Chapter 14. Interfaces

Chapter 1. Project Overview

Design, implement and synthesize a modular RV32I processor core using a classical five-stage in-order pipeline (IF, ID, EX, MEM, WB). The processor targets implementation on the Digilent Basys 3 (Artix-7) FPGA and serves both as a study of processor microarchitecture and as a portfolio-quality RTL design project.

Design Goals

- Correctly execute the supported RV32I instruction subset (defined in Chapter 3).

- Maintain strict separation between control logic (Control Unit), data-path (ALU, register file, immediate generator), and pipeline sequencing (pipeline registers).

- Handle data hazards via forwarding and stalling; handle control hazards via flushing (mechanism detailed in Chapter 10).

- Prioritize clean, readable, modular RTL over micro-optimized or highly compressed logic.

- Ensure each pipeline stage communicates exclusively through well-defined pipeline registers.

Supported ISA

RV32I Base Integer ISA restricted to the instruction subset specified in Chapter 3.

Template

Architectural Template:

Single-core, Single-issue, In-order execution

Harvard memory architecture, Ideal instruction and data memories

Implementation Template:

RTL written in synthesizable Verilog

FPGA implementation performed using Vivado 2025.2

Target platform: Digilent Basys 3 (Artix-7 XC7A35T)

Success Criteria

The processor shall:

- Execute every supported instruction correctly.

- Correctly resolve RAW hazards using forwarding.

- Correctly stall on load-use hazards.

- Correctly flush the pipeline after taken branches and jumps.

- Produce correct architectural state for all supported test programs.

- Pass module-level and processor-level verification.

- Synthesize successfully on the Basys 3 FPGA.

Chapter 2. Overall Architecture

2.1 Microarchitectural Design Decisions

Each instruction progresses sequentially through the five pipeline stages. Multiple instructions may reside simultaneously in different stages, allowing instruction-level parallelism while maintaining in-order execution.

2.2 Pipeline Organization

Instruction execution is divided into five stages:

- Instruction Fetch (IF)

Fetch instruction from instruction memory  Compute the sequential PC (PC + 4)  Select the next program counter  Store fetched information in the IF/ID pipeline register

- Instruction Decode (ID)

Decode instruction fields  Read operands from the register file  Generate immediate values  Generate processor control signals  Detect hazards before execution

- Execute (EX)

Select operands through forwarding and operand multiplexers  Perform arithmetic and logical operations  Evaluate branch conditions  Compute memory addresses  Compute branch target addresses

- Memory Access (MEM)

Perform data memory read/write operations  Forward execution results toward write-back

- Write Back (WB)

Select the correct write-back source  Update the destination register

Each stage is separated by a dedicated pipeline register.

2.3 Architectural Decisions

- What architecture should this processor have?

Possible options:

* Single-cycle			* Multi-cycle		* Pipelined

I chose a **5-stage pipelined architecture**. Important considerations are:

- A single-cycle processor has a very long critical path because every instruction must complete in one clock cycle.

- A multi-cycle processor improves clock frequency but introduces a finite-state controller and variable instruction latency.

- A pipelined processor overlaps instruction execution, allowing multiple instructions to be in flight simultaneously. It represents the architecture used by classical RISC processors and is the natural next step after studying the single-cycle data-path.

- Why exactly five stages?

Because the work naturally divides into five distinct tasks.

IF → ID → EX → MEM → WB

Each stage performs one major operation. This separation keeps each stage relatively simple and allows a higher clock frequency than a single-cycle implementation.

It is also the pipeline organization described in Hennessy & Patterson, making it easier to verify my implementation against a standard reference.

- Why in-order execution?

Alternatives:

* Out-of-order

I selected **in-order execution** because:

* simpler hardware           			* no register renaming

* no reorder buffer				* no reservation stations

Since the purpose of this project is to understand RTL implementation rather than maximize IPC, in-order execution provides the best balance between complexity and learning value.

- Why single issue?

One instruction is issued every cycle (under ideal conditions).

Alternatives:

* superscalar			* VLIW		* multiple issue

Multiple issue would require dependency checking between simultaneously issued instructions, multiple execution units, and a much more complex control structure.

That complexity is beyond the scope of this project.

- Why Harvard architecture?

Alternative:	* Von Neumann

I selected Harvard architecture because instruction fetch and data memory access occur simultaneously in a pipelined processor. If both shared one memory, a structural hazard would occur whenever IF and MEM required memory in the same cycle. Using separate instruction and data memories removes this hazard completely. It also maps naturally onto FPGA block RAM resources.

- Why no cache?

Because cache design is an independent topic. The purpose of this project is processor microarchitecture.

Adding caches would introduce

* cache controller					* replacement policy

* cache misses						* stall logic

- without changing the core pipeline itself.

- Why no branch prediction?

Branches are resolved in the Execute stage. If a branch is taken, younger instructions are flushed.

Alternatives:

* static prediction 					* dynamic prediction

* BTB							* BHT

These techniques improve performance but greatly increase control complexity.

Since correctness is the primary objective, I chose explicit flushing.

- Why forwarding?

Without forwarding, almost every data dependency introduces pipeline stalls. Forwarding allows results to be used before they are written back, significantly reducing CPI with relatively little hardware. The only remaining unavoidable hazard is the load-use hazard.

- Why stall after a load?

Example:

lw x5,0(x1)

add x6,x5,x7

The loaded data is not available until the MEM stage. Forwarding cannot provide data that has not yet been produced. Therefore, a one-cycle stall is unavoidable.

- Why modular RTL?

Instead of writing one large processor module, every hardware block will be implemented independently.

Benefits:

* Easier debugging				* Easier verification

* Reusable modules				* Clearer interfaces

Each module will have its own RTL, testbench, and waveform verification before integration.

Chapter 3. Supported Instruction Set

3.1 Design Choice

The processor implements a subset of the RV32I Base Integer Instruction Set. The objective is to implement a processor capable of executing the fundamental classes of RISC-V instructions while avoiding additional complexity associated with advanced ISA features (e.g., multiplication/division, compressed instructions, CSR operations, interrupts, and exceptions).

The selected instruction subset exercises every major hardware component of the processor:

*ALU operations					*Immediate generation

*Register file access					*Memory interface

*Branch and jump control				*Program counter update

*Write-back logic

3.2 Execution Units

R-Type

Instruction 	Execution Unit 		Operation

ADD		ALU			ADD

SUB		ALU			SUB

AND		ALU			AND

OR		ALU			OR

XOR		ALU			XOR

SLL		ALU			Shift Left Logical

SRL		ALU			Shift Right Logical

SRA		ALU			Shift Right Arithmetic

SLT		ALU			Signed Less Than

SLTU		ALU			Unsigned Less Than

I-Type

Instruction  	Execution Unit		Operation

ADDI		ALU			ADD

ANDI		ALU			AND

ORI		ALU			OR

XORI		ALU			XOR

SLLI		ALU			Shift Left Logical

SRLI		ALU			Shift Right Logical

SRAI		ALU			Shift Right Arithmetic

SLTI		ALU			Signed Less Than

LW		ALU			Address Calculation (Base + Offset)

JALR		ALU			Target Address Calculation (Base + Offset)

S-Type

Instruction	Execution Unit		Operation

SW		ALU	       		Address Calculation (Base + Offset)

B-Type

Instruction	Execution Unit		Operation

BEQ		Comparator		Equality Comparison

BNE		Comparator		Inequality Comparison

BLT		Comparator		Signed Less Than

BGE		Comparator		Signed Greater Than or Equal

U-Type

Instruction	Execution Unit		Operation

LUI		ALU			Pass Immediate (PASS_B)

AUIPC		ALU			PC + Immediate

J-Type

Instruction	Execution Unit		Operation

JAL		Branch Adder		PC + Immediate (Target), PC+4 for Link

3.3 Instructions Not Implemented

The following RV32I instructions are intentionally omitted:

Load Instructions LB LH LBU LHU 		Store Instructions SB SH

Unsigned Branches BLTU BGEU 			System Instructions ECALL EBREAK

3.4 Motivation for ISA Selection

The objective of ISA selection was not to implement every instruction defined in the RV32I specification, but rather to implement a subset that exercises every major hardware block in the processor while avoiding additional hardware whose architectural concepts are already represented by other instructions.

- Why implement immediate instructions?

Immediate instructions verify that the processor correctly supports:

Immediate generation, Sign extension, ALU operand selection, Immediate arithmetic

- Why implement only LW and SW?

Memory instructions are required to verify:

Address calculation				Data memory interface

Memory read/write control			Write-back from memory

A single load (LW) and a single store (SW) are sufficient to validate these architectural features.

- Why not implement LB, LH, LBU and LHU?

These instructions do not introduce a new architectural concept.

Instead, they require additional implementation complexity such as:

Byte and half-word extraction			Byte-enable logic

Sign extension					Zero extension

Alignment handling

Since these mechanisms are independent of the processor pipeline itself, they were deferred.

- Why not implement SB and SH?

SB and SH require partial-word writes into memory.

Supporting these instructions introduces:

Byte-enable generation				Write masking

Additional memory interface logic

These features do not change the processor microarchitecture and can be added later without modifying the pipeline.

- Why implement branches?

Conditional branches are essential for verifying:

Comparator operation				Branch decision logic

Branch target calculation			Pipeline flushing

Control hazard handling

Without branch instructions, the processor would never exercise control-flow changes.

- Why implement only BEQ, BNE, BLT and BGE?

These four instructions provide complete coverage of the comparator hardware implemented in this processor. Unsigned branches (BLTU and BGEU) require unsigned comparison logic. Since signed comparison already validates the branch data-path and hazard handling mechanisms, unsigned variants were omitted from the initial implementation.

- Why implement JAL and JALR?

These instructions verify:

*Program counter redirection		*Link register update

*Write-back of PC+4			*Function call support

Together they exercise both absolute and register-based control transfers.

- Why implement LUI and AUIPC?

- Although these instructions are relatively simple, they verify two important data-path operations. LUI validates immediate handling by routing the U-type immediate through the ALU using the PASS_B operation before write-back. This avoids introducing a dedicated immediate write-back path and keeps the write-back stage uniform.

- AUIPC validates PC-relative arithmetic by adding the U-type immediate to the current program counter.

- Rather than introducing dedicated hardware for simple instructions, the design preferentially reuses existing data-path resources whenever doing so does not significantly increase control complexity. The implementation of LUI using the ALU PASS_B operation is an example of this philosophy.

- Together, these instructions complete the processor's immediate handling capability while reusing the existing ALU data-path.

- Why exclude ECALL and EBREAK?

These instructions belong to the system-level functionality of the RISC-V architecture.

Supporting them requires:

*Exception handling			*Trap vectors

*Privileged architecture			*Machine control registers (CSRs)

These features are outside the scope of this project.

- Why not implement the entire RV32I ISA?

The objective of this project is to design a correct and modular pipelined processor, not to maximize ISA coverage. Instruction selection was driven by hardware coverage rather than instruction count. Every implemented instruction exercises at least one unique data-path or control feature, while omitted instructions primarily represent refinements or extensions of already-implemented hardware rather than new architectural concepts.

Chapter 4. Data-path

The IF stage begins with the pc signal, which is the current Program Counter value stored in the PC register and represents the byte address of the instruction currently being fetched. Since the PC is an architectural state element, it remains constant throughout the entire clock cycle and only changes on the active clock edge if the PC register is enabled. This signal fans out to multiple modules simultaneously: it is sent to the instruction memory to fetch the current instruction, to the PC+4 adder to compute the sequential next address, and is also forwarded to the IF/ID pipeline register so that the Decode stage retains the original address associated with the fetched instruction even after the PC advances in subsequent cycles. The pc_4 signal is a purely combinational value generated by adding four to the current PC because RV32I uses fixed-length 32-bit (4-byte) instructions, making the sequential next instruction address equal to PC + 4; this value is later required by instructions such as JAL, which write the return address (PC + 4) into the destination register, so it is computed once in the Fetch stage and propagated through the pipeline rather than recomputed later. The instruction signal is the 32-bit instruction fetched asynchronously from the instruction memory using the current PC as the address, specifically by indexing the instruction memory with pc[11:2], since the memory is organized as 1024 words rather than bytes and the lower two address bits are always zero due to 4-byte alignment. The next_pc signal represents the address that will be loaded into the Program Counter at the next rising clock edge and is therefore the result of the PC selection logic rather than an architectural state itself; it exists only during the current combinational evaluation and disappears after the PC register captures it. The pc_sel signal is a 2-bit control signal generated by the pc_control module, which determines which candidate address should become the next PC by selecting between the sequential address (PC + 4), the branch or JAL target address, and the ALU result for JALR instructions; unlike data signals, pc_sel belongs entirely to the control path and never represents processor data. The pc_enable signal is generated by the hazard detection unit and controls whether the Program Counter register is allowed to update on the next clock edge; when asserted, the PC advances normally, while when deasserted, the PC retains its previous value, effectively freezing instruction fetch during a pipeline stall. The if_id_enable signal similarly controls the IF/ID pipeline register and determines whether the fetched instruction packet is allowed to enter the Decode stage; if it is deasserted, the IF/ID register simply retains its previous contents, preventing the Decode stage from observing a new instruction during hazards. The if_id_flush signal is used to invalidate the contents of the IF/ID pipeline register, typically after a taken branch or jump has redirected control flow, by replacing the fetched instruction packet with zeros so that the incorrectly fetched instruction is discarded before it reaches Decode. Finally, the signals if_id_pc, if_id_pc_4, and if_id_instruction are the outputs of the IF/ID pipeline register and together form the complete instruction packet transferred from the Fetch stage to the Decode stage. These outputs are stable for the entire Decode cycle because they are stored in flip-flops, allowing the Decode stage to work on a consistent snapshot of the fetched instruction even while the Fetch stage is already processing the next instruction. Together, these signals separate naturally into architectural state (pc), combinational datapath values (pc_4, instruction, next_pc), control signals (pc_sel, pc_enable, if_id_enable, if_id_flush), and registered pipeline outputs (if_id_pc, if_id_pc_4, if_id_instruction), with each category playing a distinct role in ensuring that instruction fetch, control-flow selection, pipeline synchronization, and hazard handling all occur correctly every clock cycle.

Chapter 5. Pipeline Registers

Pipeline registers carry only the data and decoded control information required by downstream stages. The original instruction is discarded after decode unless a later stage explicitly requires one of its fields. So we will not carry forward the whole instruction, Propagate the full instruction only through IF/ID (input to decode). From ID/EX onward, propagate only the decoded fields required downstream FLUSH is not necessarily required on every pipeline register. Once an instruction has reached EX or MEM, it is generally allowed to complete, only younger instructions are flushed. So IF/ID and ID/EX need stall and FLUSH.

The EX/MEM and MEM/WB pipeline registers include an enable input for interface uniformity. In the current processor implementation these enables remain permanently asserted because instructions that have entered the Execute stage are always allowed to complete.

5.1 Design Decisions

- Why is the complete instruction propagated only through IF/ID?

Decision

The complete 32-bit instruction is propagated only through the IF/ID pipeline register. Beginning with ID/EX, only the decoded operands, register addresses, immediate value, and control signals are propagated.

- Why?

Instruction decoding is performed only once in the Decode stage. Re-decoding the instruction in later stages would introduce unnecessary hardware and duplicate logic. Carrying only the required information reduces the amount of data stored in the pipeline registers and makes each stage independent of the instruction encoding.

- Trade-off

The original instruction is no longer available in later pipeline stages for debugging or tracing. Recovering the instruction would require additional debug logic if needed.

- Why are source and destination register addresses propagated?

Decision: The decoded register addresses (RS1_ADDR, RS2_ADDR, and RD_ADDR) are propagated through the pipeline instead of the complete instruction.

- Why?

These fields are required by the forwarding and hazard detection logic to identify data dependencies between instructions. Propagating only the register indices provides the required information while avoiding the overhead of carrying the entire instruction.

- Why is the immediate propagated only until the Execute stage?

Decision: The immediate value is propagated through the ID/EX register only.

- Why?

The ALU uses the immediate as an operand during the Execute stage. Once the ALU has produced its result, the original immediate value is no longer required. For instructions such as LUI, the ALU performs a PASS_B operation, eliminating the need for a dedicated immediate write-back path.

- Benefits

- Narrower EX/MEM and MEM/WB pipeline registers.

- Simpler write-back multiplexer.

- More uniform data-path.

- Why do only IF/ID and ID/EX support flushing?

Decision: Flush capability is implemented only in the IF/ID and ID/EX pipeline registers.

Why?

Branches are resolved during the Execute stage. At that instant:

- Instructions in IF and ID are younger than the branch and must be discarded if the branch is taken.

- Instructions in MEM and WB are older than the branch and must complete to preserve precise program execution.

- Flushing later pipeline stages would incorrectly discard valid instructions.

- Why do EX/MEM and MEM/WB include an enable signal?

Decisions:

All pipeline registers implement a common interface that includes an enable input. Why?

Although EX/MEM and MEM/WB are never stalled in the current processor, providing a uniform interface simplifies module reuse, improves readability, and allows future architectural extensions without modifying the module interface.

In the current implementation, these enable inputs remain permanently asserted.

- Why does LUI no longer require an immediate write-back path?

LUI is implemented as an ALU PASS_B operation. The immediate is selected as the second ALU operand and forwarded unchanged to the ALU result. Consequently, the write-back stage receives the value through the existing ALU result path, eliminating the need for a separate immediate write-back source.

- Why is PC+4 propagated through three pipeline registers?

PC+4 is required for the write-back of JAL and JALR. Since write-back occurs in the WB stage, the value must be preserved as the instruction progresses through the pipeline.

- Why is the original PC propagated only to ID/EX?

The original program counter is only needed during the Execute stage for operations such as AUIPC and branch target address calculation (PC + immediate). Once execution is complete, the original PC has no further purpose and is therefore not propagated.

Chapter 6. Control Signals

6.1 Purpose of the Control Unit

The control unit interprets the decoded instruction and generates the control signals required by the data-path. These signals determine operand selection, ALU operation, memory access, write-back source, branch behaviour, and register updates. Once generated in the Decode stage, the control signals are propagated through the pipeline registers until they are no longer required.

6.2 Control Signal Summary

Signal			Width	Stage Generated		Used In

IMM_SEL		3	ID			Immediate Generator

OP1_SEL		1	ID			ALU Input MUX

OP2_SEL		1	ID			ALU Input MUX

ALU_OP		5	ID			ALU

BRANCH_JUMP		3	ID			Branch Unit

MEM_READ		1	ID			Data Memory

MEM_WRITE		1	ID			Data Memory

WB_SEL		2	ID			Writeback MUX

REG_WRITE_EN		1	ID			Register File

IMM_SEL

Purpose Selects the immediate format extracted from the instruction.

Encoding

IMM_SEL	Immediate

000		R (None)

001		I

010		S

011		B

100		U

101		J

110		Reserved

111		Reserved

OP1_SEL

Purpose: Selects ALU operand A.

Encoding

Value	Operand A

0	RS1

1	PC

OP2_SEL

Purpose: Selects ALU operand B.

Encoding

Value	Operand B

0	RS2

1	Immediate

ALU_OP

Purpose: select ALU operations.

Encoding

ALU_OP	Operation

00000		ADD

00001		SUB

00010		AND

00011		OR

00100		XOR

00101		SLL

00110		SRL

00111		SRA

01000		SLT

01001		SLTU

01010		PASS_B

01011–11111	Reserved

BRANCH_JUMP

Purpose: select type of Branching.

Encoding

BRANCH_JUMP		Meaning

000			No Branch/Jump

001			BEQ

010			BNE

011			BLT

100			BGE

101			JAL

110			JALR

111			Reserved

MEM_READ

Purpose: Enables a read operation from the data memory.

Encoding

MEM_READ	Operation

0		No memory read

1		Read data memory

MEM_WRITE

Purpose: Enables a write operation to the data memory.

Encoding

MEM_WRITE	Operation

0		No memory write

1		Write data memory

WB_SEL

Purpose: selecting which source to take.

Encoding

WB_SEL	Source

00		ALU Result

01		Data Memory

10		PC+4

11		Reserved

REG_WRITE_EN

Purpose: selecting Reg.

Encoding

Value		Meaning

0		No Register Write

1		Enable Register Write

Not all of these Control signals will be used till end of cycle

signal		Last Pipeline Stage Used

IMM_SEL		ID

OP1_SEL		EX

OP2_SEL		EX

ALU_OP		EX

BRANCH_JUMP		EX

MEM_READ		MEM

MEM_WRITE		MEM

WB_SEL		WB

REG_WRITE_EN		WB

Why generate all control signals in Decode? Decode is the only stage that interprets the instruction, avoiding repeated decoding downstream.

Why use fixed-width encoded signals? Encoded control buses reduce wiring and make interfaces scalable compared with individual one-hot control lines.

Why keep reserved encodings? They support future ISA extensions without altering the module interfaces.

- Why are encoded control signals used?

Decision

Operations such as ALU functions and branch types are represented using encoded control signals rather than individual one-hot control lines.

Why?

Encoded signals reduce the number of wires between modules and simplify the interfaces. They also provide reserved encodings for future ISA extensions without modifying the processor architecture.

- Why are MEM_READ and MEM_WRITE separate signals?

Decision

Memory read and write operations are controlled using two independent one-bit signals. Why?

The memory interface closely follows the classical data-path described in Hennessy and Patterson, where read and write operations are independently controlled. This makes the data-path easier to understand and simplifies the memory control logic for the supported instruction subset (LW and SW).

- Why isn't IMM_SEL propagated beyond the Decode stage?

IMM_SEL is only required by the Immediate Generator to determine how the immediate field should be extracted from the instruction. Once the 32-bit immediate has been generated, the selection information has served its purpose and is no longer needed.

Chapter 7. ALU Specification

7.1 Purpose

The Arithmetic Logic Unit (ALU) is the primary computational unit of the processor. It performs arithmetic, logical, comparison, shift, address generation, and operand forwarding operations during the Execute stage. The specific operation is selected by the ALU_OP control signal generated in the Decode stage.

7.2 ALU Interface

Inputs

Signal		Width	Description

OPERAND_A	32	First ALU operand

OPERAND_B	32	Second ALU operand

ALU_OP	5	Operation select

Outputs

Signal		Width	Description

ALU_RESULT	32	Result of selected operation

We are not using status flags (Zero, Carry, Overflow, etc.) because our branch decisions are handled by a dedicated comparator. That is one of our architectural choices.

7.3 Supported Operations

ALU_OP	Operation	Description

00000		ADD		Addition

00001		SUB		Subtraction

00010		AND		Bitwise AND

00011		OR		Bitwise OR

00100		XOR		Bitwise XOR

00101		SLL		Logical left shift

00110		SRL		Logical right shift

00111		SRA		Arithmetic right shift

01000		SLT		Signed comparison

01001		SLTU		Unsigned comparison

01010		PASS_B		Forward operand B unchanged

01011		Reserved	Future extensions

–11111

7.4 Operand Selection

Instead of giving the ALU many input ports, the operand multiplexers select the operands before they reach the ALU.

Operand A

OP1_SEL	Source

0		RS1_DATA

1		PC

Operand B

OP2_SEL	Source

0		RS2_DATA

1		Immediate

7.5 Instructions Using the ALU

ALU Operation	Used By

ADD		ADD, ADDI, LW, SW, AUIPC, JALR

SUB		SUB

AND		AND, ANDI

OR		OR, ORI

XOR		XOR, XORI

SLL		SLL, SLLI

SRL		SRL, SRLI

SRA		SRA, SRAI

SLT		SLT, SLTI

SLTU		SLTU

PASS_B		LUI

7.6 Design Decisions

- Why does the ALU perform address calculations?

Address generation is mathematically identical to integer addition. Reusing the ALU for effective address calculation avoids dedicated address-generation hardware while simplifying the data-path.

- Why is PASS_B included?

The PASS_B operation allows the ALU to forward its second operand unchanged. This operation is used by the LUI instruction, enabling immediate values to be written back through the standard ALU result path instead of requiring a dedicated immediate write-back path.

- Why doesn't the ALU perform branch comparisons?

Branch comparison is handled by a dedicated comparator.

Separating comparison from arithmetic allows the branch unit and ALU to operate independently and simplifies branch control logic.

- Why doesn't the ALU compute branch targets?

Branch target addresses are generated by a dedicated branch target adder.

This allows arithmetic operations, branch comparison, and target address generation to be treated as separate functional units within the Execute stage.

- Why are shift operations implemented in the ALU instead of using a separate shifter?

Shift operations are part of the RV32I integer instruction set and naturally belong within the integer execution unit. Incorporating the barrel shifter into the ALU keeps the data-path compact and avoids introducing an additional execution unit.

Chapter 8. Immediate Generator

8.1 Purpose

The Immediate Generator extracts the immediate field from the fetched instruction and converts it into a 32-bit value suitable for arithmetic, address generation, and control-flow instructions. The immediate format is selected using the IMM_SEL control signal generated during the Decode stage.

Immediate Generation

We will look at what each type does,

(refer to Instruction formats)

I-type: Extract immediate from bits [31:20] and sign-extend.

S-type: Concatenate bits [31:25] and [11:7], then sign-extend.

B-type: Rearrange branch offset bits, append an implicit zero LSB, and sign-extend.

U-type: Place the upper 20 bits in bits [31:12] and clear the lower 12 bits.

J-type: Rearrange jump offset bits, append an implicit zero LSB, and sign-extend.

8.2 Interface

Inputs

Signal		Width	Description

INSTRUCTION	32	Current instruction

IMM_SEL	3	Immediate format selector

Outputs

Signal		Width	Description

IMMEDIATE	32	Generated immediate

8.3 Supported Immediate Formats

(same table from 6.2)

IMM_SEL	Format		Used By

000		R		No immediate

001		I		ADDI, ANDI, ORI, XORI, SLLI, SRLI, SRAI, LW, JALR, SLTI

010		S		SW

011		B		BEQ, BNE, BLT, BGE

100		U		LUI, AUIPC

101		J		JAL

110	    	Reserved	—

111	    	Reserved	—

8.4 Design Decisions

- Why use a dedicated Immediate Generator?

Decision

A dedicated combinational module generates all immediate values.

Why?

Centralizing immediate extraction avoids duplicating bit-manipulation logic throughout the data-path and allows all instructions requiring immediates to use a common interface.

- Why use IMM_SEL?

The immediate encoding differs among instruction formats. The IMM_SEL control signal selects the appropriate extraction logic while keeping the module interface simple.

- Why generate the immediate during Decode?

Immediate extraction depends only on the instruction bits. Performing this operation during the Decode stage ensures that the Execute stage receives a fully formed 32-bit operand without additional decoding.

- Why is an Immediate Generator required?

RISC-V stores immediate values differently across instruction formats. A dedicated Immediate Generator converts these various encodings into a uniform 32-bit value that can be used by the Execute stage.

- Why doesn't the ALU generate the immediate?

The ALU performs arithmetic and logical operations on operands that have already been prepared. Immediate extraction is a decoding task based solely on instruction bit fields and therefore belongs in the Decode stage.

- Which instructions do not use the Immediate Generator?

Pure register-register (R-type) instructions do not require an immediate operand. For these instructions, IMM_SEL selects the R-type format, and the generated immediate is ignored.

Chapter 9. Branch and Control-Flow Unit

9.1 Purpose

The Branch Unit is responsible for evaluating branch conditions and generating PC-relative target addresses. It operates during the Execute stage and consists of three independent functional units: a Comparator, Branch Target Adder, PC Control Logic.

Arithmetic execution and control-flow execution are implemented using separate hardware resources, Our processor deliberately separates these functions into three execution resources:

1) ALU

2) Comparator

3) Branch Target Adder

Whenever the next Program Counter differs from the sequential value (PC + 4), a flush request is asserted. The IF/ID and ID/EX pipeline registers are flushed so that instruction fetching resumes from the correct target address.

This Chapter describes the hardware responsible for control-flow execution, which consists of:

Comparator (EX stage)                 evaluates the branch condition.

Branch Target Adder (EX stage)  computes PC + Immediate.

Next-PC Multiplexer (IF stage)    selects the next program counter.

9.2 Branch Unit Organization

Comparator: Responsible for evaluating branch conditions.

Supports:

BEQ				BNE

BLT				BGE

Branch Target Adder: Responsible for computing Target Address = PC + Immediate

Used by

BEQ				BNE

BLT				BGE

JAL				(# JALR is done by ALU's ADD instruction)

Although referred to collectively as the Branch Unit, the control-flow hardware is distributed across the pipeline. The Execute stage performs branch evaluation and target address generation, while the Instruction Fetch stage updates the Program Counter by selecting the appropriate next PC through a multiplexer.

9.3 Branch Unit Interface

Comparator

Inputs

Signal			Width	Description

RS1_DATA		32	First operand

RS2_DATA		32	Second operand

BRANCH_JUMP		3	Branch type

Outputs

Signal			Width	Description

BRANCH_TAKEN	1	Branch decision

Branch Target Adder

Inputs

Signal			Width

PC			32

IMMEDIATE		32

Outputs

Signal			Width

TARGET_ADDRESS	32

9.4 Supported Operations

Comparator

BRANCH_JUMP		Operation

000			No branch/jump

001			BEQ

010			BNE

011			BLT

100			BGE

101			JAL

110			JALR

111			Reserved

Target Address Generation

Instruction		Target Computation

BEQ			PC + Immediate

BNE			PC + Immediate

BLT			PC + Immediate

BGE			PC + Immediate

JAL			PC + Immediate

9.5 Branch Unit Outputs

Output			Width		Purpose

BRANCH_TAKEN	1		Branch comparison result

TARGET_ADDRESS	32		PC-relative target address

9.6 PC Control Logic

Function

Determines the source of the next Program Counter value using the decoded instruction type and the branch decision.

Inputs

Signal			Width

BRANCH_JUMP		3

BRANCH_TAKEN	1

Outputs

Signal			Width

PC_SEL			2

9.7 Next PC Selection (for PC mux)

The PC selection logic chooses between four possible values.

Condition			Next PC

000 (Normal)			PC + 4

001–100 (Branch taken)		Branch Target Adder Output

001–100 (Branch not taken)	PC + 4

101 (JAL)			Branch Target Adder Output

110 (JALR)			ALU Result

9.8 Design Decisions

- Why use a dedicated comparator?

Decision

Branch comparisons are performed by a dedicated comparator rather than the ALU.

Why?

Branch comparison is logically independent of arithmetic operations. A dedicated comparator simplifies ALU design and separates arithmetic from control-flow evaluation.

Trade-off

Additional combinational hardware.

- Why use a dedicated Branch Target Adder?

Decision

Target addresses for PC-relative branches and jumps are computed using a dedicated adder.

Why?

Target address calculation is independent of arithmetic instructions. A dedicated adder allows branch target computation without overloading the ALU.

Trade-off

Slight increase in hardware area.

- Why is JALR handled differently?

Decision

JALR target addresses are computed by the ALU.

Why?

Unlike branches and JAL, JALR uses a register value (RS1) instead of the current program counter as the base address. Since the ALU already supports base-plus-offset addition, no additional hardware is required. JAL computes its target as PC + Immediate, which matches the Branch Target Adder's function. JALR computes its target as RS1 + Immediate

- Why are branches resolved in the Execute stage?

Decision

All branch decisions are completed during the Execute stage.

Why?

Both the comparison result and the target address become available in EX. Resolving branches earlier would require moving comparison hardware into Decode, while resolving them later would unnecessarily increase the branch penalty.

- What happens when a branch is taken?

The comparator asserts BRANCH_TAKEN, the PC multiplexer selects the branch target address, and the younger instructions in the IF and ID stages are flushed. Instructions already in EX, MEM, and WB continue normally.

- Why doesn't the Branch Unit write to the register file?

The Branch Unit only determines control flow. Register updates for JAL and JALR are performed through the normal write-back path using PC+4, while ordinary branch instructions do not produce a register result.

Chapter 10. Hazard Handling

10.1 Purpose

In a pipelined processor, multiple instructions execute simultaneously in different pipeline stages. This overlap can introduce situations where an instruction cannot proceed correctly because the required data or resources are unavailable. The Hazard Handling Unit ensures correct program execution by detecting and resolving such hazards while minimizing performance penalties.

10.2 Types of Pipeline Hazards

Hazard Type		Present?	Resolution

Structural Hazard	No		Eliminated using Harvard Architecture

Data Hazard		Yes		Forwarding + One-cycle Stall

Control Hazard		Yes		Pipeline Flush

- Structural Hazard

Decision

Not present.

Why?

Instruction fetch and data memory access use separate memories.

- Data Hazard

Occurs when an instruction depends on the result of an earlier instruction.

Resolved by

Forwarding				Load-use stall

- Control Hazard

Occurs when branches or jumps change the PC.

Resolved by

Branch resolution in EX			Flush IF/ID and ID/EX

10.3 Forwarding Unit

Purpose

The Forwarding Unit eliminates unnecessary pipeline stalls by supplying the most recent result directly from later pipeline stages instead of waiting for register write-back.

Inputs

Signal				Width

ID/EX.RS1_ADDR		5

ID/EX.RS2_ADDR		5

EX/MEM.RD_ADDR		5

MEM/WB.RD_ADDR		5

EX/MEM.REG_WRITE_EN	1

MEM/WB.REG_WRITE_EN	1

Outputs

Signal		Width

Forward_A	2

Forward_B	2

Forwarding sources

Value		Source

00		Register File

01		MEM/WB

10		EX/MEM

11		Reserved

10.4 Load-Use Hazard Detection

(Unavoidable stall)

Forwarding cannot help because memory data is not available until the MEM stage.

Therefore, we need to:

Stall IF				Stall ID

Insert bubble into EX  That is Penalty of 1 Cycle

Inputs Signal

ID/EX.MEM_READ

ID/EX.RD_ADDR

IF/ID.RS1_ADDR

IF/ID.RS2_ADDR

Outputs Signal

PC_ENABLE

IF_ID_ENABLE

ID_EX_FLUSH

10.5 Control Hazard Handling

Branches resolved in EX

If branch taken then Flush IF/ID, Flush ID/EX, Load PC with target.

No prediction.

10.6 Hazard Resolution Summary

Hazard			Resolution	Penalty

ALU dependency	Forwarding	0 cycles

Load-use		1 Stall		1 cycle

Taken Branch		Flush		2 instructions

Structural		None		0

10.7 Design Decisions

- Why use forwarding?

Without forwarding, almost every dependent ALU instruction would require pipeline stalls, significantly reducing throughput.

- Why is load-use still stalled?

Forwarding cannot provide data before the memory access has completed. Since the loaded value becomes available only after the MEM stage, one pipeline bubble is unavoidable.

- Why no branch prediction?

The project prioritizes architectural clarity and modular RTL over aggressive performance optimization. Branch prediction would substantially increase design complexity while contributing little to the educational objectives.

- Why are structural hazards eliminated instead of stalled?

Using separate instruction and data memories removes contention for memory access, simplifying the hazard handling logic and improving pipeline throughput.

- Which hazards exist in your processor?

The processor encounters data hazards and control hazards. Structural hazards are eliminated through the Harvard memory architecture.

- Why can't forwarding solve a load-use hazard?

The result of a load instruction is produced only after the memory access completes in the MEM stage. The following instruction requires the operand at the beginning of the EX stage, one cycle earlier than the data becomes available. Consequently, a single-cycle stall is unavoidable.

- Why is forwarding possible for ALU instructions?

The ALU result is available at the end of the Execute stage and can be forwarded directly to the next instruction without waiting for register write-back.

- Why are only IF and ID stalled during a load-use hazard?

The load instruction already in EX must continue progressing to complete the memory access. Stalling the younger instructions while allowing the load to advance inserts exactly one pipeline bubble and preserves correct execution.

- Why do you flush IF/ID and ID/EX instead of EX/MEM?

When a branch is resolved in EX, the instructions in IF and ID belong to the wrong control path and must be discarded. Instructions in MEM and WB are older than the branch and therefore belong to the correct execution path.

- How many forwarding paths does your processor have?

Processor implements four data forwarding paths to eliminate most RAW (Read-After-Write) data hazards without stalling the pipeline. Two forwarding paths originate from the EX/MEM pipeline register and two originate from the MEM/WB pipeline register. Specifically, the processor supports EX/MEM → ALU Operand A, EX/MEM → ALU Operand B, MEM/WB → ALU Operand A, and MEM/WB → ALU Operand B. The forwarding unit compares the source register addresses (id_ex_rs1_addr and id_ex_rs2_addr) of the instruction currently in the Execute stage against the destination register addresses (ex_mem_rd_addr and mem_wb_rd_addr) of instructions in the EX/MEM and MEM/WB pipeline registers. If a match is found and the corresponding instruction writes to the register file (reg_write_en = 1) while the destination register is not x0, the forwarding unit generates the appropriate control signals (forwardA and forwardB) to select the forwarded value instead of the value read from the register file. EX/MEM forwarding is given higher priority than MEM/WB forwarding because it contains the most recently computed result. These four forwarding paths resolve ALU-to-ALU dependencies in consecutive instructions, while load-use hazards still require a one-cycle stall since the loaded data becomes available only after the Memory stage.

10.8 Hazard Handling Hardware

Hardware			Function

Forwarding Unit		Resolves ALU data hazards

Hazard Detection Unit		Detects load-use hazards and generates stall signals

Pipeline Control Logic		Generates ENABLE and FLUSH signals

Branch Unit			Resolves control hazards

Chapter 11. Memory System

11.1 Purpose

The Memory System provides instruction storage and data storage for the processor. A Harvard architecture is adopted, using separate instruction and data memories to eliminate structural hazards between instruction fetch and data access.

11.2 Memory Organization

- completely independent memories

- independent address buses

- independent data buses

- Completely Independent Memories

There are two separate memories:

- Instruction Memory stores program instructions.

- Data Memory stores variables and other data.

Since they are separate, accessing one does not affect the other.

- Independent Address Buses

An address bus carries the address of the memory location to be accessed.

Harvard architecture has:

- One address bus for Instruction Memory.

- Another address bus for Data Memory.

This allows the CPU to send instruction and data addresses at the same time.

- Independent Data Buses

A data bus transfers data between the CPU and memory.

There are two separate data buses:

- One transfers instructions from Instruction Memory to the CPU.

- The other transfers data between Data Memory and the CPU.

Both transfers can occur simultaneously.

11.3 Instruction Memory

Stores the program instructions.

Inputs

Signal			Width

Address			32

Outputs

Signal			Width

Instruction		32

Characteristics

Read-only during execution.

Accessed only during IF.

One 32-bit instruction per access.

11.4 Data Memory

Stores program data.

Inputs

Signal			Width

Address			32

Write Data		32

MEM_READ		1

MEM_WRITE		1

Outputs

Signal			Width

Read Data		32

Characteristics

Supports LW.

Supports SW.

One word per access.

11.5 Supported Operations

Instruction		Memory Operation

LW			Read Word

SW			Write Word

11.6 Memory Access Flow

IF:

PC  Instruction Memory   Instruction

Load:

ALU  Address  Data Memory  Read Data  WB

Store:

RS2  Data Memory Write Port  Memory

11.7 Design Decisions

- Why no cache?

The processor is intended as an educational RTL implementation. Introducing caches would significantly increase design complexity while obscuring the core pipeline concepts.

- Why single-cycle memories?

Ideal single-cycle memories simplify pipeline timing and allow the project to focus on data-path and control logic rather than memory latency.

- Why only word accesses?

The implemented ISA subset contains only LW and SW. Supporting byte and halfword accesses would require additional alignment logic, byte-enable circuitry, and sign/zero extension, none of which introduce fundamentally new architectural concepts for this project.

Chapter 12. Register File

12.1 Purpose

The Register File stores the processor's architectural registers and provides operands for instruction execution. It supports simultaneous reading of two source registers and writing of one destination register, allowing register-to-register and register-immediate instructions to execute efficiently within the five-stage pipeline.

12.2 Organization

Parameter		Value

Number of Registers	32

Register Width		32 bits

Read Ports		2

Write Ports		1

Register x0		Constant Zero

12.3 Interface

Inputs

Signal			Width		Description

RS1_ADDR		5		Read Address 1

RS2_ADDR		5		Read Address 2

RD_ADDR		5		Write Address

WRITE_DATA		32		Data to be written

REG_WRITE_EN		1		Write Enable

CLK			1		Clock

RESET			1		Reset

Outputs

Signal			Width		Description

RS1_DATA		32		Read Data 1

RS2_DATA		32		Read Data 2

12.4 Read and Write Operation

Read

- Two registers can be read simultaneously using the source register addresses supplied by the instruction.

- Reads are combinational.

Write

- One register is written during the Write-Back stage.

- Writes occur on the active clock edge when REG_WRITE_EN = 1

12.5 Register x0

RISC-V defines register x0 as a hardwired zero register.

Characteristics:

Always contains zero.

Reads always return zero.

Writes are ignored.

This behaviour is enforced by the Register File implementation.

If software attempts to write to x0, Then write is ignored. Register x0 is hardwired to zero and always returns zero regardless of any attempted write operation.

Register	Purpose

x0		Constant zero (hardwired)

x1–x31		General-purpose registers

12.6 Design Decisions

- Why two read ports?

Most instructions require two source operands. Two independent read ports allow both operands to be read simultaneously during the Decode stage.

- Why only one write port?

The processor is single-issue and retires at most one instruction per cycle. Therefore only one register write can occur during each clock cycle.

- Why are reads combinational?

Combinational reads allow operand values to become available immediately during the Decode stage, enabling execution in the following cycle without introducing additional pipeline latency.

- Why are writes synchronous?

Synchronous writes ensure that register updates occur only on clock edges, preventing race conditions and maintaining predictable pipeline timing.

- Why is only one write port sufficient?

The processor is a single-issue, in-order pipeline that completes at most one instruction per cycle. Consequently, only one architectural register can be updated during each write-back stage.

- What would change if the processor became dual-issue?

A dual-issue processor could retire two instructions in the same cycle, requiring at least two write ports and additional read ports. The increased port count significantly increases the complexity and area of the Register File.

12.7 Architectural Register Set

The processor implements the standard RV32I architectural register set consisting of thirty-two 32-bit integer registers. Although all registers except x0 are identical in hardware, the RISC-V ABI assigns conventional software roles to each register.

The stack pointer (sp, register x2) points to the current top of the program's stack, which stores function call information such as local variables, saved registers, and return addresses.

return address register

When JAL ra, function is executed, the processor writes PC + 4 into x1 (ra) Later, JALR x0, ra, 0

returns to the caller.

x2 called the stack pointer software (compiler/ABI) decides that x2 will contain the address of the stack (not enforced by Hardware)

x10–x17 (a0–a7) are Function arguments / return values

Chapter 13. Module List

Pipeline Stage		Modules

IF	Program Counter (PC), PC + 4 Adder, PC Selection Multiplexer, PC Control Logic, Instruction Memory, IF/ID Pipeline Register

ID	Register File, Immediate Generator, Main Control Unit, Hazard Detection Unit, ID/EX Pipeline Register

EX	Forwarding Unit, Operand Selection Multiplexers, Arithmetic Logic Unit (ALU), Branch Unit, EX/MEM Pipeline Register

MEM		Data Memory, MEM/WB Pipeline Register

WB		Writeback Multiplexer

Final module count

Stage		Number of Modules

IF		6

ID		5

EX		5

MEM		2

WB		1

Total	19 Modules

(The Module list will be in appendix)

Chapter 14. Interfaces

14.1 Processor Top-Level Interface

Function

Defines the external interface of the RV32I processor. These signals connect the processor to instruction memory, data memory, reset circuitry, and the system clock.

Clock and Reset

Signal	Direction	Width	Description

CLK	Input		1	System clock

RESET	Input		1	Processor reset

Instruction Memory Interface

Signal			Direction	Width	Description

IMEM_ADDRESS	Output		32	Instruction address

IMEM_DATA		Input		32	Instruction word

Data Memory Interface

Signal			Direction	Width	Description

DMEM_ADDRESS	Output		32	Memory address

DMEM_WRITE_DATA	Output		32	Data to store

DMEM_READ_DATA	Input		32	Loaded data

DMEM_READ		Output		1	Read enable

DMEM_WRITE		Output		1	Write enable

Hardware Summary

Processor type: RV32I

Pipeline: Five-stage

Instruction width: 32 bits

Data width: 32 bits

Address width: 32 bits

Register count: 32

14.2 RTL implementation

module pc(

input clk,

input reset,

input enable,

input [31:0] next_pc,

output reg [31:0] pc

);

always @(posedge clk)

begin

if (reset)

pc <= 32'd0;

else if (enable)

pc <= next_pc;

end

endmodule

`timescale 1ns/1ps

module pc_mux (

input  wire [31:0] pc_4,

input  wire [31:0] target_address,

input  wire [31:0] alu_result,

input  wire [1:0] pc_sel,

output reg  [31:0] next_pc

);

always @(*) begin

case (pc_sel)

2'b00: next_pc = pc_4;

2'b01: next_pc = target_address;

2'b10: next_pc = alu_result;

default: next_pc = pc_4;

endcase

end

endmodule

`timescale 1ns/1ps

module pc_4_adder (

input  wire [31:0] pc,

output wire [31:0] pc_4

);

assign pc_4 = pc + 32'd4;

endmodule

`timescale 1ns/1ps

module pc_mux (

input  wire [31:0] pc_4,

input  wire [31:0] target_address,

input  wire [31:0] alu_result,

input  wire [1:0] pc_sel,

output reg  [31:0] next_pc

);

always @(*) begin

case (pc_sel)

2'b00: next_pc = pc_4;

2'b01: next_pc = target_address;

2'b10: next_pc = alu_result;

default: next_pc = pc_4;

endcase

end

endmodule

`timescale 1ns/1ps

module if_id_pipeline (

input  wire        clk,

input  wire        reset,

input  wire        enable,

input  wire        flush,

input  wire [31:0] pc_in,

input  wire [31:0] pc_4_in,

input  wire [31:0] instruction_in,

output reg  [31:0] pc_out,

output reg  [31:0] pc_4_out,

output reg  [31:0] instruction_out

);

always @(posedge clk or posedge reset) begin

if (reset) begin

pc_out          <= 32'b0;

pc_4_out        <= 32'b0;

instruction_out <= 32'b0;

end

else if (flush) begin

pc_out          <= 32'b0;

pc_4_out        <= 32'b0;

instruction_out <= 32'b0;

end

else if (enable) begin

pc_out          <= pc_in;

pc_4_out        <= pc_4_in;

instruction_out <= instruction_in;

end

end

endmodule

`timescale 1ns/1ps

module hazard_detection_unit (

input  wire [4:0] id_rs1_addr,

input  wire [4:0] id_rs2_addr,

input  wire [4:0] id_ex_rd_addr,

input  wire       id_ex_mem_read,

output reg        pc_enable,

output reg        if_id_enable,

output reg        id_ex_flush

);

always @(*) begin

if (id_ex_mem_read && (id_ex_rd_addr != 5'd0) &&

((id_ex_rd_addr == id_rs1_addr) || (id_ex_rd_addr == id_rs2_addr)))

begin

pc_enable    = 1'b0;

if_id_enable = 1'b0;

id_ex_flush  = 1'b1;

end

else begin

pc_enable    = 1'b1;

if_id_enable = 1'b1;

id_ex_flush  = 1'b0;

end

end

endmodule

`timescale 1ns/1ps

module immediate_generator (

input  wire [31:0] instruction,

input  wire [2:0]  imm_sel,

output reg  [31:0] imm

);

always @(*) begin

case (imm_sel)

// R-Type

3'b000:  imm = 32'b0;

// I-Type

3'b001:  imm = {{20{instruction[31]}}, instruction[31:20]};

// S-Type

3'b010: imm = {{20{instruction[31]}},  instruction[31:25],instruction[11:7]};

// B-Type

3'b011: imm = {{19{instruction[31]}},instruction[31],instruction[7],instruction[30:25],instruction[11:8],1'b0};

// U-Type

3'b100: imm = {instruction[31:12], 12'b0};

// J-Type

3'b101: imm = {{11{instruction[31]}},instruction[31],instruction[19:12], instruction[20],instruction[30:21],

1'b0};

default: imm = 32'b0;

endcase

end

endmodule

`timescale 1ns/1ps

module register_file (

input  wire        clk,

input  wire        reset,

input  wire        reg_write_en,

input  wire [4:0]  rs1_addr,

input  wire [4:0]  rs2_addr,

input  wire [4:0]  rd_addr,

input  wire [31:0] write_data,

output wire [31:0] rs1_data,

output wire [31:0] rs2_data

);

reg [31:0] registers [0:31];

integer i;

// Synchronous write

always @(posedge clk or posedge reset) begin

if (reset) begin

for (i = 0; i < 32; i = i + 1)

registers[i] <= 32'b0;

end

else if (reg_write_en && (rd_addr != 5'd0)) begin

registers[rd_addr] <= write_data;

end

end

// Asynchronous read

assign rs1_data = (rs1_addr == 5'd0) ? 32'b0 : registers[rs1_addr];

assign rs2_data = (rs2_addr == 5'd0) ? 32'b0 : registers[rs2_addr];

endmodule

`timescale 1ns/1ps

module main_control_unit (

input  wire [6:0] opcode,

input  wire [2:0] funct3,

input  wire [6:0] funct7,

output reg  [2:0] imm_sel,

output reg        op1_sel,

output reg        op2_sel,

output reg  [4:0] alu_op,

output reg  [2:0] branch_jump,

output reg        mem_read,

output reg        mem_write,

output reg  [1:0] wb_sel,

output reg        reg_write_en

);

localparam R_TYPE = 7'b0110011;

localparam I_TYPE = 7'b0010011;

localparam LOAD   = 7'b0000011;

localparam STORE  = 7'b0100011;

localparam BRANCH = 7'b1100011;

localparam LUI    = 7'b0110111;

localparam AUIPC  = 7'b0010111;

localparam JAL    = 7'b1101111;

localparam JALR   = 7'b1100111;

localparam ADD    = 5'b00000;

localparam SUB    = 5'b00001;

localparam AND_OP = 5'b00010;

localparam OR_OP  = 5'b00011;

localparam XOR_OP = 5'b00100;

localparam SLL    = 5'b00101;

localparam SRL    = 5'b00110;

localparam SRA    = 5'b00111;

localparam SLT    = 5'b01000;

localparam SLTU   = 5'b01001;

localparam PASS_B = 5'b01010;

always @(*) begin

imm_sel      = 3'b000;

op1_sel      = 1'b0;      // RS1

op2_sel      = 1'b0;      // RS2

alu_op       = ADD;

branch_jump  = 3'b000;

mem_read     = 1'b0;

mem_write    = 1'b0;

wb_sel       = 2'b00;     // ALU Result

reg_write_en = 1'b0;

case (opcode)

R_TYPE: begin

reg_write_en = 1'b1;

case (funct3)

3'b000: alu_op = (funct7 == 7'b0100000) ? SUB : ADD;

3'b001: alu_op = SLL;

3'b010: alu_op = SLT;

3'b011: alu_op = SLTU;

3'b100: alu_op = XOR_OP;

3'b101: alu_op = (funct7 == 7'b0100000) ? SRA : SRL;

3'b110: alu_op = OR_OP;

3'b111: alu_op = AND_OP;

default: alu_op = ADD;

endcase

end

I_TYPE: begin

imm_sel      = 3'b001;

op2_sel      = 1'b1;

reg_write_en = 1'b1;

case (funct3)

3'b000: alu_op = ADD;                         // ADDI

3'b001: alu_op = SLL;                         // SLLI

3'b010: alu_op = SLT;                         // SLTI

3'b011: alu_op = SLTU;                        // SLTIU

3'b100: alu_op = XOR_OP;                      // XORI

3'b101: alu_op = (funct7 == 7'b0100000) ? SRA : SRL;

3'b110: alu_op = OR_OP;                       // ORI

3'b111: alu_op = AND_OP;                      // ANDI

default: alu_op = ADD;

endcase

end

LOAD: begin

imm_sel      = 3'b001;

op2_sel      = 1'b1;

alu_op       = ADD;

mem_read     = 1'b1;

wb_sel       = 2'b01;

reg_write_en = 1'b1;

end

STORE: begin

imm_sel   = 3'b010;

op2_sel   = 1'b1;

alu_op    = ADD;

mem_write = 1'b1;

end

BRANCH: begin

imm_sel = 3'b011;

case (funct3)

3'b000: branch_jump = 3'b001;     // BEQ

3'b001: branch_jump = 3'b010;     // BNE

3'b100: branch_jump = 3'b011;     // BLT

3'b101: branch_jump = 3'b100;     // BGE

default: branch_jump = 3'b000;

endcase

end

LUI: begin

imm_sel      = 3'b100;

op2_sel      = 1'b1;

alu_op       = PASS_B;

reg_write_en = 1'b1;

end

AUIPC: begin

imm_sel      = 3'b100;

op1_sel      = 1'b1;

op2_sel      = 1'b1;

alu_op       = ADD;

reg_write_en = 1'b1;

end

JAL: begin

imm_sel      = 3'b101;

branch_jump  = 3'b101;

wb_sel       = 2'b10;

reg_write_en = 1'b1;

end

JALR: begin

imm_sel      = 3'b001;

op2_sel      = 1'b1;

alu_op       = ADD;

branch_jump  = 3'b110;

wb_sel       = 2'b10;

reg_write_en = 1'b1;

end

default: begin  // Keep default control signals

end

endcase

end

endmodule

`timescale 1ns/1ps

module id_ex_pipeline(

input  wire [31:0] pc_in,

input  wire [31:0] pc_4_in,

input  wire [31:0] rs1_data_in,

input  wire [31:0] rs2_data_in,

input  wire [31:0] imm_in,

input  wire [4:0]  rs1_addr_in,

input  wire [4:0]  rs2_addr_in,

input  wire [4:0]  rd_addr_in,

input  wire        op1_sel_in,

input  wire        op2_sel_in,

input  wire [4:0]  alu_op_in,

input  wire [2:0]  branch_jump_in,

input  wire        mem_read_in,

input  wire        mem_write_in,

input  wire [1:0]  wb_sel_in,

input  wire        reg_write_en_in,

input  wire        clk,

input  wire        reset,

input  wire        enable,

input  wire        flush,

output reg [31:0] pc_out,

output reg [31:0] pc_4_out,

output reg [31:0] rs1_data_out,

output reg [31:0] rs2_data_out,

output reg [31:0] imm_out,

output reg [4:0]  rs1_addr_out,

output reg [4:0]  rs2_addr_out,

output reg [4:0]  rd_addr_out,

output reg        op1_sel_out,

output reg        op2_sel_out,

output reg [4:0]  alu_op_out,

output reg [2:0]  branch_jump_out,

output reg        mem_read_out,

output reg        mem_write_out,

output reg [1:0]  wb_sel_out,

output reg        reg_write_en_out

);

always @(posedge clk or posedge reset) begin

if (reset) begin

pc_out           <= 32'b0;

pc_4_out         <= 32'b0;

rs1_data_out     <= 32'b0;

rs2_data_out     <= 32'b0;

imm_out          <= 32'b0;

rs1_addr_out     <= 5'b0;

rs2_addr_out     <= 5'b0;

rd_addr_out      <= 5'b0;

op1_sel_out      <= 1'b0;

op2_sel_out      <= 1'b0;

alu_op_out       <= 5'b0;

branch_jump_out  <= 3'b0;

mem_read_out     <= 1'b0;

mem_write_out    <= 1'b0;

wb_sel_out       <= 2'b0;

reg_write_en_out <= 1'b0;

end

else if (flush) begin

pc_out           <= 32'b0;

pc_4_out         <= 32'b0;

rs1_data_out     <= 32'b0;

rs2_data_out     <= 32'b0;

imm_out          <= 32'b0;

rs1_addr_out     <= 5'b0;

rs2_addr_out     <= 5'b0;

rd_addr_out      <= 5'b0;

op1_sel_out      <= 1'b0;

op2_sel_out      <= 1'b0;

alu_op_out       <= 5'b0;

branch_jump_out  <= 3'b0;

mem_read_out     <= 1'b0;

mem_write_out    <= 1'b0;

wb_sel_out       <= 2'b0;

reg_write_en_out <= 1'b0;

end

else if (enable) begin

pc_out           <= pc_in;

pc_4_out         <= pc_4_in;

rs1_data_out     <= rs1_data_in;

rs2_data_out     <= rs2_data_in;

imm_out          <= imm_in;

rs1_addr_out     <= rs1_addr_in;

rs2_addr_out     <= rs2_addr_in;

rd_addr_out      <= rd_addr_in;

op1_sel_out      <= op1_sel_in;

op2_sel_out      <= op2_sel_in;

alu_op_out       <= alu_op_in;

branch_jump_out  <= branch_jump_in;

mem_read_out     <= mem_read_in;

mem_write_out    <= mem_write_in;

wb_sel_out       <= wb_sel_in;

reg_write_en_out <= reg_write_en_in;

end

end

endmodule

`timescale 1ns/1ps

module alu (

input  wire [31:0] operand_a,

input  wire [31:0] operand_b,

input  wire [4:0]  alu_op,

output reg [31:0] alu_result

);

localparam ADD    = 5'b00000;

localparam SUB    = 5'b00001;

localparam AND_OP = 5'b00010;

localparam OR_OP  = 5'b00011;

localparam XOR_OP = 5'b00100;

localparam SLL    = 5'b00101;

localparam SRL    = 5'b00110;

localparam SRA    = 5'b00111;

localparam SLT    = 5'b01000;

localparam SLTU   = 5'b01001;

localparam PASS_B = 5'b01010;

always @(*) begin

case (alu_op)

ADD:alu_result = operand_a + operand_b;

SUB: alu_result = operand_a - operand_b;

AND_OP: alu_result = operand_a & operand_b;

OR_OP: alu_result = operand_a | operand_b;

XOR_OP: alu_result = operand_a ^ operand_b;

SLL: alu_result = operand_a << operand_b[4:0];

SRL: alu_result = operand_a >> operand_b[4:0];

SRA: alu_result = $signed(operand_a) >>> operand_b[4:0];

SLT:alu_result = ($signed(operand_a) < $signed(operand_b)) ? 32'd1 : 32'd0;

SLTU: alu_result = (operand_a < operand_b) ? 32'd1 : 32'd0;

PASS_B: alu_result = operand_b;

default: alu_result = 32'b0;

endcase

end

endmodule

`timescale 1ns/1ps

module branch_comparator (

input  wire [31:0] rs1_data,

input  wire [31:0] rs2_data,

input  wire [2:0]  branch_jump,

output reg         branch_taken

);

always @(*) begin

case (branch_jump)

// BEQ     3'b001:branch_taken = (rs1_data == rs2_data);

// BNE     3'b010:branch_taken = (rs1_data != rs2_data);

// BLT      3'b011:branch_taken = ($signed(rs1_data) < $signed(rs2_data));

// BGE    3'b100:branch_taken = ($signed(rs1_data) >= $signed(rs2_data));

// Not a conditional branch

default:branch_taken = 1'b0;

endcase

end

endmodule

`timescale 1ns/1ps

module branch_target_adder (

input  wire [31:0] pc,

input  wire [31:0] imm,

output wire [31:0] target_address

);

assign target_address = pc + imm;

endmodule

`timescale 1ns/1ps

module forwarding_unit (

input  wire [4:0] id_ex_rs1_addr,

input  wire [4:0] id_ex_rs2_addr,

input  wire [4:0] ex_mem_rd_addr,

input  wire [4:0] mem_wb_rd_addr,

input  wire       ex_mem_reg_write_en,

input  wire       mem_wb_reg_write_en,

output reg [1:0] forwardA,

output reg [1:0] forwardB

);

always @(*) begin

forwardA = 2'b00;

forwardB = 2'b00;

if (ex_mem_reg_write_en &&   (ex_mem_rd_addr != 5'd0) &&

(ex_mem_rd_addr == id_ex_rs1_addr))

begin

forwardA = 2'b10;

end

else if (mem_wb_reg_write_en &&(mem_wb_rd_addr != 5'd0) &&

(mem_wb_rd_addr == id_ex_rs1_addr))

begin

forwardA = 2'b01;

end

if (ex_mem_reg_write_en && (ex_mem_rd_addr != 5'd0) &&

(ex_mem_rd_addr == id_ex_rs2_addr))

begin

forwardB = 2'b10;

end

else if (mem_wb_reg_write_en &&(mem_wb_rd_addr != 5'd0) &&

(mem_wb_rd_addr == id_ex_rs2_addr))

begin

forwardB = 2'b01;

end

end

endmodule

`timescale 1ns/1ps

module operand_selection_muxes (

input  wire [31:0] rs1_data,

input  wire [31:0] rs2_data,

input  wire [31:0] imm,

input  wire [31:0] pc,

input  wire [31:0] ex_mem_alu_result,

input  wire [31:0] mem_wb_write_data,

input  wire        op1_sel,

input  wire        op2_sel,

input  wire [1:0]  forwardA,

input  wire [1:0]  forwardB,

output reg [31:0] operand_a,

output reg [31:0] operand_b

);

reg [31:0] rs1_forwarded;

reg [31:0] rs2_forwarded;

always @(*) begin

case (forwardA)

2'b00: rs1_forwarded = rs1_data;

2'b01: rs1_forwarded = mem_wb_write_data;

2'b10: rs1_forwarded = ex_mem_alu_result;

default: rs1_forwarded = rs1_data;

endcase

end

always @(*) begin

case (forwardB)

2'b00: rs2_forwarded = rs2_data;

2'b01: rs2_forwarded = mem_wb_write_data;

2'b10: rs2_forwarded = ex_mem_alu_result;

default: rs2_forwarded = rs2_data;

endcase

end

always @(*) begin

case (op1_sel)

1'b0: operand_a = rs1_forwarded;

1'b1: operand_a = pc;

endcase

end

always @(*) begin

case (op2_sel)

1'b0: operand_b = rs2_forwarded;

1'b1: operand_b = imm;

endcase

end

endmodule

`timescale 1ns/1ps

module ex_mem_pipeline (

input  wire [31:0] pc_4_in,

input  wire [31:0] alu_result_in,

input  wire [31:0] rs2_data_in,

input  wire [4:0]  rd_addr_in,

input  wire        mem_read_in,

input  wire        mem_write_in,

input  wire [1:0]  wb_sel_in,

input  wire        reg_write_en_in,

input  wire        clk,

input  wire        reset,

input  wire        enable,

output reg [31:0] pc_4_out,

output reg [31:0] alu_result_out,

output reg [31:0] rs2_data_out,

output reg [4:0]  rd_addr_out,

output reg        mem_read_out,

output reg        mem_write_out,

output reg [1:0]  wb_sel_out,

output reg        reg_write_en_out

);

always @(posedge clk or posedge reset) begin

if (reset) begin

pc_4_out         <= 32'b0;

alu_result_out   <= 32'b0;

rs2_data_out     <= 32'b0;

rd_addr_out      <= 5'b0;

mem_read_out     <= 1'b0;

mem_write_out    <= 1'b0;

wb_sel_out       <= 2'b0;

reg_write_en_out <= 1'b0;

end

else if (enable) begin

pc_4_out         <= pc_4_in;

alu_result_out   <= alu_result_in;

rs2_data_out     <= rs2_data_in;

rd_addr_out      <= rd_addr_in;

mem_read_out     <= mem_read_in;

mem_write_out    <= mem_write_in;

wb_sel_out       <= wb_sel_in;

reg_write_en_out <= reg_write_en_in;

end

end

endmodule

`timescale 1ns/1ps

module data_memory (

input  wire [31:0] alu_result,

input  wire [31:0] write_data,

input  wire        mem_read,

input  wire        mem_write,

input  wire        clk,

output reg [31:0]  dmem_data

);

reg [31:0] memory [0:1023];

always @(posedge clk) begin

if (mem_write)

memory[alu_result[11:2]] <= write_data;

end

always @(*) begin

if (mem_read)

dmem_data = memory[alu_result[11:2]];

else

dmem_data = 32'b0;

end

endmodule

`timescale 1ns/1ps

module mem_wb_pipeline (

input  wire [31:0] pc_4_in,

input  wire [31:0] alu_result_in,

input  wire [31:0] dmem_data_in,

input  wire [4:0]  rd_addr_in,

input  wire [1:0]  wb_sel_in,

input  wire        reg_write_en_in,

input  wire        clk,

input  wire        reset,

input  wire        enable,

output reg [31:0] pc_4_out,

output reg [31:0] alu_result_out,

output reg [31:0] dmem_data_out,

output reg [4:0]  rd_addr_out,

output reg [1:0]  wb_sel_out,

output reg        reg_write_en_out

);

always @(posedge clk or posedge reset) begin

if (reset) begin

pc_4_out         <= 32'b0;

alu_result_out   <= 32'b0;

dmem_data_out    <= 32'b0;

rd_addr_out      <= 5'b0;

wb_sel_out       <= 2'b0;

reg_write_en_out <= 1'b0;

end

else if (enable) begin

pc_4_out         <= pc_4_in;

alu_result_out   <= alu_result_in;

dmem_data_out    <= dmem_data_in;

rd_addr_out      <= rd_addr_in;

wb_sel_out       <= wb_sel_in;

reg_write_en_out <= reg_write_en_in;

end

end

endmodule

`timescale 1ns/1ps

module writeback_mux (

input  wire [31:0] alu_result,

input  wire [31:0] dmem_data,

input  wire [31:0] pc_4,

input  wire [1:0]  wb_sel,

output reg  [31:0] write_data

);

always @(*) begin

case (wb_sel)

2'b00: write_data = alu_result;

2'b01: write_data = dmem_data;

2'b10: write_data = pc_4;

default: write_data = 32'b0;

endcase

end

endmodule

`timescale 1ns/1ps

module cpu(

input  wire clk,

input  wire reset,

output wire [15:0] led

);

wire [31:0] pc;

wire [31:0] next_pc;

wire [31:0] pc_4;

wire [31:0] instruction;

wire [1:0]  pc_sel;

wire        pc_enable;      // from hazard detection unit (stall)

wire        if_id_enable;   // from hazard detection unit (stall)

wire        if_id_flush;    // branch/jump squash

wire [31:0] if_id_pc;

wire [31:0] if_id_pc_4;

wire [31:0] if_id_instruction;

wire [4:0] rs1_addr;

wire [4:0] rs2_addr;

wire [4:0] rd_addr;

wire [6:0] opcode;

wire [2:0] funct3;

wire [6:0] funct7;

wire [31:0] rs1_data;

wire [31:0] rs2_data;

wire [31:0] imm;

wire       op1_sel;

wire       op2_sel;

wire [4:0] alu_op;

wire [2:0] branch_jump;

wire       mem_read;

wire       mem_write;

wire [1:0] wb_sel;

wire       reg_write_en;

wire [2:0] imm_sel;

wire       id_ex_flush;

wire [31:0] id_ex_pc;

wire [31:0] id_ex_pc_4;

wire [31:0] id_ex_rs1_data;

wire [31:0] id_ex_rs2_data;

wire [31:0] id_ex_imm;

wire [4:0] id_ex_rs1_addr;

wire [4:0] id_ex_rs2_addr;

wire [4:0] id_ex_rd_addr;

wire       id_ex_op1_sel;

wire       id_ex_op2_sel;

wire [4:0] id_ex_alu_op;

wire [2:0] id_ex_branch_jump;

wire       id_ex_mem_read;

wire       id_ex_mem_write;

wire [1:0] id_ex_wb_sel;

wire       id_ex_reg_write_en;

wire [1:0] forwardA;

wire [1:0] forwardB;

wire [31:0] operand_a;

wire [31:0] operand_b;

wire [31:0] alu_result;

wire        branch_taken;

wire        branch_flush

wire [31:0] target_address;

wire [31:0] ex_mem_pc_4;

wire [31:0] ex_mem_alu_result;

wire [31:0] ex_mem_rs2_data;

wire [4:0] ex_mem_rd_addr;

wire       ex_mem_mem_read;

wire       ex_mem_mem_write;

wire [1:0] ex_mem_wb_sel;

wire       ex_mem_reg_write_en;

wire [31:0] dmem_data;

wire [31:0] mem_wb_pc_4;

wire [31:0] mem_wb_alu_result;

wire [31:0] mem_wb_dmem_data;

wire [4:0] mem_wb_rd_addr;

wire [1:0] mem_wb_wb_sel;

wire       mem_wb_reg_write_en;

wire [31:0] write_data;   // also used as mem_wb_write_data forwarding source

assign branch_flush = branch_taken && (id_ex_branch_jump != 3'b000);

assign if_id_flush  = branch_flush;

assign opcode   = if_id_instruction[6:0];

assign funct3   = if_id_instruction[14:12];

assign funct7   = if_id_instruction[31:25];

assign rs1_addr = if_id_instruction[19:15];

assign rs2_addr = if_id_instruction[24:20];

assign rd_addr  = if_id_instruction[11:7];

assign led = alu_result[15:0];

pc pc_reg (

.next_pc (next_pc),

.clk     (cpu_clk),

.reset   (reset),

.enable  (pc_enable),

.pc      (pc)

);

pc_4_adder pc_4_adder_inst (

.pc   (pc),

.pc_4 (pc_4)

);

pc_control pc_control_inst (

.branch_jump  (id_ex_branch_jump),

.branch_taken (branch_taken),

.pc_sel       (pc_sel)

);

pc_mux pc_mux_inst (

.pc_4           (pc_4),

.target_address (target_address),

.alu_result     (alu_result),

.pc_sel         (pc_sel),

.next_pc        (next_pc)

);

instruction_memory imem_inst (

.pc          (pc),

.instruction (instruction)

);

if_id_pipeline if_id_reg (

.pc_in          (pc),

.pc_4_in        (pc_4),

.instruction_in (instruction),

.clk            (cpu_clk),

.reset          (reset),

.enable         (if_id_enable),

.flush          (if_id_flush),

.pc_out         (if_id_pc),

.pc_4_out       (if_id_pc_4),

.instruction_out(if_id_instruction)

);

register_file rf_inst (

.rs1_addr     (rs1_addr),

.rs2_addr     (rs2_addr),

.rd_addr      (mem_wb_rd_addr),

.write_data   (write_data),

.reg_write_en (mem_wb_reg_write_en),

.clk          (cpu_clk),

.reset        (reset),

.rs1_data     (rs1_data),

.rs2_data     (rs2_data)

);

immediate_generator imm_gen_inst (

.instruction (if_id_instruction),

.imm_sel     (imm_sel),

.imm         (imm)

);

main_control_unit ctrl_inst (

.opcode       (opcode),

.funct3       (funct3),

.funct7       (funct7),

.imm_sel      (imm_sel),

.op1_sel      (op1_sel),

.op2_sel      (op2_sel),

.alu_op       (alu_op),

.branch_jump  (branch_jump),

.mem_read     (mem_read),

.mem_write    (mem_write),

.wb_sel       (wb_sel),

.reg_write_en (reg_write_en)

);

wire id_ex_flush_hazard;

hazard_detection_unit hdu_inst (

.id_rs1_addr    (rs1_addr),

.id_rs2_addr    (rs2_addr),

.id_ex_rd_addr  (id_ex_rd_addr),

.id_ex_mem_read (id_ex_mem_read),

.pc_enable      (pc_enable),

.if_id_enable   (if_id_enable),

.id_ex_flush    (id_ex_flush_hazard)

);

// combine hazard-driven bubble with branch-squash

assign id_ex_flush = id_ex_flush_hazard || branch_flush;

id_ex_pipeline id_ex_reg (

.pc_in            (if_id_pc),

.pc_4_in          (if_id_pc_4),

.rs1_data_in      (rs1_data),

.rs2_data_in      (rs2_data),

.imm_in           (imm),

.rs1_addr_in      (rs1_addr),

.rs2_addr_in      (rs2_addr),

.rd_addr_in       (rd_addr),

.op1_sel_in       (op1_sel),

.op2_sel_in       (op2_sel),

.alu_op_in        (alu_op),

.branch_jump_in   (branch_jump),

.mem_read_in      (mem_read),

.mem_write_in     (mem_write),

.wb_sel_in        (wb_sel),

.reg_write_en_in  (reg_write_en),

.clk              (cpu_clk),

.reset            (reset),

.enable           (1'b1),

.flush            (id_ex_flush),

.pc_out           (id_ex_pc),

.pc_4_out         (id_ex_pc_4),

.rs1_data_out     (id_ex_rs1_data),

.rs2_data_out     (id_ex_rs2_data),

.imm_out          (id_ex_imm),

.rs1_addr_out     (id_ex_rs1_addr),

.rs2_addr_out     (id_ex_rs2_addr),

.rd_addr_out      (id_ex_rd_addr),

.op1_sel_out      (id_ex_op1_sel),

.op2_sel_out      (id_ex_op2_sel),

.alu_op_out       (id_ex_alu_op),

.branch_jump_out  (id_ex_branch_jump),

.mem_read_out     (id_ex_mem_read),

.mem_write_out    (id_ex_mem_write),

.wb_sel_out       (id_ex_wb_sel),

.reg_write_en_out (id_ex_reg_write_en)

);

forwarding_unit fwd_inst (

.id_ex_rs1_addr      (id_ex_rs1_addr),

.id_ex_rs2_addr      (id_ex_rs2_addr),

.ex_mem_rd_addr      (ex_mem_rd_addr),

.mem_wb_rd_addr      (mem_wb_rd_addr),

.ex_mem_reg_write_en (ex_mem_reg_write_en),

.mem_wb_reg_write_en (mem_wb_reg_write_en),

.forwardA            (forwardA),

.forwardB            (forwardB)

);

operand_selection_muxes op_mux_inst (

.rs1_data          (id_ex_rs1_data),

.rs2_data          (id_ex_rs2_data),

.imm               (id_ex_imm),

.pc                (id_ex_pc),

.ex_mem_alu_result (ex_mem_alu_result),

.mem_wb_write_data (write_data),

.op1_sel           (id_ex_op1_sel),

.op2_sel           (id_ex_op2_sel),

.forwardA          (forwardA),

.forwardB          (forwardB),

.operand_a         (operand_a),

.operand_b         (operand_b)

);

alu alu_inst (

.operand_a  (operand_a),

.operand_b  (operand_b),

.alu_op     (id_ex_alu_op),

.alu_result (alu_result)

);

branch_comparator branch_cmp_inst (

.rs1_data(operand_a),

.rs2_data(operand_b),

.branch_jump(id_ex_branch_jump),

.branch_taken(branch_taken)

);

branch_target_adder branch_adder_inst (

.pc             (id_ex_pc),

.imm            (id_ex_imm),

.target_address (target_address)

);

ex_mem_pipeline ex_mem_reg (

.pc_4_in          (id_ex_pc_4),

.alu_result_in    (alu_result),

.rs2_data_in      (id_ex_rs2_data),

.rd_addr_in       (id_ex_rd_addr),

.mem_read_in      (id_ex_mem_read),

.mem_write_in     (id_ex_mem_write),

.wb_sel_in        (id_ex_wb_sel),

.reg_write_en_in  (id_ex_reg_write_en),

.clk              (cpu_clk),

.reset            (reset),

.enable           (1'b1),

.pc_4_out         (ex_mem_pc_4),

.alu_result_out   (ex_mem_alu_result),

.rs2_data_out     (ex_mem_rs2_data),

.rd_addr_out      (ex_mem_rd_addr),

.mem_read_out     (ex_mem_mem_read),

.mem_write_out    (ex_mem_mem_write),

.wb_sel_out       (ex_mem_wb_sel),

.reg_write_en_out (ex_mem_reg_write_en)

);

data_memory dmem_inst (

.alu_result (ex_mem_alu_result),

.write_data (ex_mem_rs2_data),   // store data, not the wb write_data

.mem_read   (ex_mem_mem_read),

.mem_write  (ex_mem_mem_write),

.clk        (cpu_clk),

.dmem_data  (dmem_data)

);

mem_wb_pipeline mem_wb_reg (

.pc_4_in          (ex_mem_pc_4),

.alu_result_in    (ex_mem_alu_result),

.dmem_data_in     (dmem_data),

.rd_addr_in       (ex_mem_rd_addr),

.wb_sel_in        (ex_mem_wb_sel),

.reg_write_en_in  (ex_mem_reg_write_en),

.clk              (cpu_clk),

.reset            (reset),

.enable           (1'b1),

.pc_4_out         (mem_wb_pc_4),

.alu_result_out   (mem_wb_alu_result),

.dmem_data_out    (mem_wb_dmem_data),

.rd_addr_out      (mem_wb_rd_addr),

.wb_sel_out       (mem_wb_wb_sel),

.reg_write_en_out (mem_wb_reg_write_en)

);

writeback_mux wb_mux_inst (

.alu_result (mem_wb_alu_result),

.dmem_data  (mem_wb_dmem_data),

.pc_4       (mem_wb_pc_4),

.wb_sel     (mem_wb_wb_sel),

.write_data (write_data)

);

reg [26:0] counter;

always @(posedge clk or posedge reset) begin

if (reset)

counter <= 0;

else

counter <= counter + 1;

end

wire cpu_clk = counter[26];

endmodule

14.3 FPGA Implementation and Hardware Realization

Organization:

- Target FPGA Platform

- FPGA Design Flow

- Synthesis and Implementation

- Hardware Realization

- Resource Utilization

- Timing Closure

- Physical Implementation (Placement, Routing, Congestion)

- Bitstream Generation

- Engineering Lessons

14.3.1 FPGA Development Platform

The processor was implemented on the Digilent Basys 3 development board containing an AMD Xilinx Artix-7 XC7A35T-1CPG236C FPGA. The Artix-7 family is fabricated using a 28 nm CMOS SRAM-based process and is intended for low-power, moderate-performance digital systems. The device provides approximately 33,000 logic cells, 20,800 Slice LUTs, 41,600 flip-flops, 50 Block RAM tiles, 90 DSP48E1 slices, 32 global clock buffers, and 106 programmable I/O pins. The complete implementation was performed using AMD Vivado Design Suite 2025.2.

The Basys 3 board integrates a 100 MHz onboard oscillator, 16 user LEDs, 16 slide switches, five push buttons, a four-digit seven-segment display, USB-JTAG programming, USB-UART, VGA output, and PMOD expansion connectors, making it well suited for processor implementation and hardware validation.

14.3.2 FPGA Design Flow

Unlike software development, FPGA implementation converts a hardware description into a physical digital circuit. The complete implementation flow consists of:

Verilog RTL → RTL Elaboration → Logic Synthesis → Technology Mapping → Placement → Routing → Static Timing Analysis → Design Rule Checking (DRC) → Bitstream Generation → FPGA Programming

RTL elaboration constructs the design hierarchy, synthesis converts the behavioural Verilog into a technology-independent gate-level netlist, and technology mapping replaces generic logic with FPGA-specific primitives such as LUTs, flip-flops, carry chains, memories, and clock buffers. During implementation, placement assigns every resource to a physical location within the FPGA while routing establishes all programmable interconnections. Static timing analysis verifies that setup and hold constraints are satisfied before the configuration bitstream is generated and programmed into the FPGA.

14.3.3 Synthesis and Implementation

Although synthesis and implementation occur consecutively, they solve different engineering problems. Synthesis determines what hardware should be constructed from the RTL description, whereas implementation determines where that hardware physically resides inside the FPGA fabric.

Synthesis performs logic optimization, removes redundant circuitry, infers memories and arithmetic structures, and produces a gate-level netlist. Implementation then places every LUT, register, memory, DSP block, and routing resource within the FPGA while optimizing timing and routing congestion. Functional correctness alone is insufficient; the final placed-and-routed design must also satisfy all timing constraints.

14.3.4 Hardware Realization

Verilog is not executed by the FPGA. Instead, it serves only as a hardware description during design. After synthesis, the RTL disappears and is replaced by a netlist. Following placement and routing, the design is converted into a configuration bitstream that programs the FPGA. Once configured, the FPGA contains a physical network of lookup tables, flip-flops, routing switches, memories, carry chains, and clock networks. Consequently, an FPGA executes hardware circuits rather than software instructions, with all modules operating concurrently.

14.3.5 FPGA Resource Utilization

The completed processor occupies only a small fraction of the available FPGA resources. Vivado reported the following utilization:

| Resource | Used | Available | Utilization |
| --- | --- | --- | --- |
| LUTs | 1,484 | 20,800 | 7.13% |
| Flip-Flops | 972 | 41,600 | 2.34% |
| Distributed RAM | 512 LUT RAM | 9,600 | 5.33% |
| Block RAM | 0 | 50 | 0% |
| DSP48E1 | 0 | 90 | 0% |
| BUFG | 1 | 32 | 3.13% |
| I/O Pins | 18 | 106 | 16.98% |

The relatively low utilization indicates that substantial FPGA resources remain available for architectural extensions such as caches, multiplication units, interrupt controllers, or peripherals.

FPGA Resource Inference

Vivado automatically inferred FPGA-specific hardware structures from the behavioural Verilog description. The combinational logic of the processor—including the ALU, multiplexers, branch unit, forwarding logic, hazard detection unit, comparators, immediate generator, and control logic—was implemented using approximately 1,484 lookup tables.

Sequential storage was realized using approximately 972 flip-flops, storing the program counter, pipeline registers, register file outputs, and control signals throughout the processor.

The implementation also inferred 512 distributed RAM elements, indicating that the processor memories were implemented using LUT-based memory rather than dedicated Block RAM resources. This occurred because the memory description and access style favoured distributed RAM inference. Arithmetic operations such as addition, subtraction, PC incrementing, and branch address generation were accelerated using 50 dedicated CARRY4 primitives, while a single BUFG distributed the global system clock throughout the processor.

Vivado also generated FPGA primitives including FDRE and FDCE flip-flops, LUT1–LUT6 combinational logic elements, RAMS64E distributed memory, MUXF7 and MUXF8 multiplexers, IBUF and OBUF I/O buffers, and BUFG clock buffers, demonstrating how behavioural RTL is translated into the physical resources available on the Artix-7 architecture.

Dead Logic Removal

One important observation during development was Vivado's aggressive optimization of unused circuitry. Initially, the processor contained only clock and reset inputs without any observable outputs. Since no internal signals affected the external behaviour of the design, synthesis removed almost the entire processor as dead logic. This demonstrated that synthesis tools eliminate hardware that cannot influence observable outputs. Adding LEDs and other externally visible signals prevented this optimization and allowed the processor to remain in the synthesized design.

Design Constraints

A complete FPGA design consists of both the RTL description and the accompanying XDC constraints file. The RTL defines the hardware functionality, while the constraints specify clock definitions, physical pin assignments, I/O standards, and timing requirements. Without appropriate constraints, the synthesized logic cannot be connected to physical FPGA pins, preventing interaction with LEDs, switches, buttons, and other peripherals.

14.3.6 Timing Closure

After implementation, the processor successfully satisfied all timing requirements. The initial estimated Worst Negative Slack (WNS) was −0.138 ns, indicating a setup timing violation. Placement optimization improved WNS to 0.557 ns, and the final routed design achieved a WNS of 0.594 ns with Total Negative Slack (TNS) equal to 0 ns and a positive hold slack of 0.029 ns. These results confirm successful timing closure, demonstrating that both setup and hold timing constraints are satisfied at the target clock frequency.

14.3.7 Physical Implementation

Implementation also demonstrated successful physical optimization of the processor. Routing congestion remained extremely low, with global routing utilization of approximately 1.2%, indicating abundant routing resources within the FPGA. Before routing, Vivado reported numerous unrouted nets, which is expected because physical interconnections had not yet been assigned. After routing completed, the implementation reported zero failed nets, zero unrouted nets, and zero node overlaps, confirming that every logical connection had been successfully realized. Since timing constraints were already satisfied, Vivado determined that no additional physical optimization of the netlist was required.

14.3.8 Bitstream Generation and Programming

Following successful synthesis, implementation, timing verification, and design rule checking, Vivado generated the final FPGA configuration bitstream. Programming this bitstream through the onboard USB-JTAG interface configured the Artix-7 FPGA with the complete pipelined RV32I processor. At this stage the design ceased to exist as Verilog code and instead became a physically implemented digital circuit within the programmable logic fabric.

14.3.9 Engineering Lessons

This project demonstrated that FPGA development extends well beyond writing synthesizable Verilog. A successful implementation requires understanding the complete FPGA design flow, interpreting synthesis and implementation reports, reading resource utilization summaries, satisfying timing constraints, and performing incremental hardware bring-up. The project also illustrated how behavioral RTL is transformed into FPGA primitives, how synthesis removes unused logic, and how placement, routing, and timing optimization collectively determine whether a design can operate reliably on physical hardware. Successfully completing synthesis, implementation, timing closure, and FPGA programming provides confidence that the processor functions not only logically but also as a physically realizable hardware system.

14.4 Design Challenges, Hardware Bring-up and Engineering Lessons

14.4.1 Following design challenges were encountered during the implementation:

- RTL Integration

The processor was developed as a hierarchical RTL design consisting of approximately twenty independent Verilog modules implementing the instruction fetch, decode, execute, memory access, and write-back stages together with supporting units such as the ALU, register file, forwarding unit, hazard detection unit, branch unit, immediate generator, and control logic. As the design grew, maintaining consistent interfaces between modules became increasingly important. During integration, several issues related to signal widths, port connections, and module hierarchy were identified through Vivado elaboration and synthesis. These issues were resolved by standardizing module interfaces and carefully propagating control and data-path signals through each pipeline stage. This experience demonstrated that disciplined interface design and modular hierarchy are essential for managing large RTL projects.

- Dead Logic Removal During Synthesis

One of the most significant issues encountered during implementation occurred after synthesis completed successfully but implementation reported that the design was effectively empty. The initial processor top module contained only clock and reset inputs and produced no observable outputs. Since none of the internal logic influenced any external signal, Vivado identified the processor as functionally redundant and optimized almost the entire design away during synthesis.

To preserve the hardware, a sixteen-bit LED output was added and temporarily connected to the lower bits of the program counter. During board verification, the LEDs were later forced to a constant value (16'hFFFF) to confirm correct hardware operation. This issue illustrated how aggressively modern synthesis tools eliminate unused logic and highlighted the importance of providing observable outputs during FPGA development.

- Instruction Memory Initialization

The instruction memory was implemented as a behavioural Verilog array with asynchronous read access. Initially, the memory contained no initialization data, causing Vivado to report that the memory had no driver. The problem was resolved by adding an initial block that loaded manually encoded RV32I machine instructions into the instruction memory before synthesis. This demonstrated that FPGA ROMs require explicit initialization to generate meaningful hardware contents after configuration.

- Constraint Management

Although synthesis completed successfully, bitstream generation initially failed because the design contained no physical constraints. Vivado reported UCIO-1 and NSTD-1 violations, indicating that the clock, reset, LEDs, and other I/O signals had not been assigned to FPGA package pins or given valid electrical standards.

The Basys 3 Master XDC constraint file was imported, the required pin assignments were enabled, and the top-level Verilog port names were modified to match the constraint definitions. After applying the constraints, design rule checking succeeded and bitstream generation completed successfully. This reinforced that an FPGA design consists of both the RTL description and the physical constraint file; neither is sufficient independently.

14.4.2 Incremental Hardware Bring-Up

After generating the bitstream, the processor initially failed to produce the expected LED behaviour. Rather than attempting to debug the complete processor immediately, a systematic bring-up methodology was adopted. The LED outputs were temporarily forced to a constant logic value so that all LEDs remained continuously illuminated. This verified successful FPGA programming, correct constraint mapping, functional board hardware, and proper operation of the LED interface.

By eliminating board-level uncertainties first, the remaining debugging effort could focus exclusively on processor functionality. This experience demonstrated the importance of separating hardware integration problems from RTL logic problems during FPGA development.

- Hardware Manager Verification

During early programming attempts, Vivado Hardware Manager occasionally displayed target shutdown messages after programming the FPGA. Investigation showed that the Artix-7 device was successfully detected, configured, and reported as programmed. The shutdown messages originated from stale log entries rather than actual programming failures. This emphasized the importance of verifying device status directly rather than relying solely on console output.

- Development of a Demonstration Program

To validate processor functionality on hardware, a compact RV32I demonstration program was developed and manually encoded into machine code. The program exercised representative instruction classes including arithmetic operations (addi, add), memory access (lw, sw), conditional branching (bne), and control transfer (jal). Loading this program into the instruction ROM enabled the processor to execute a meaningful instruction sequence during FPGA demonstrations.

- Hardware Debugging Strategy

A processor operating at 100 MHz executes instructions far too rapidly for direct observation using LEDs alone. To improve hardware visibility, a debugging strategy was devised in which selected internal processor signals including the program counter, current instruction, ALU result, and write-back data would be exported through dedicated debug outputs. Combined with a programmable clock divider and manual control using switches, this approach would allow processor execution to be observed at human time scales while preserving the original RTL architecture.

14.4.3 Engineering Lessons

The implementation provided several practical insights extending beyond RTL design.

- The FPGA implementation flow consists of multiple independent stages, including elaboration, synthesis, placement, routing, timing analysis, bitstream generation, and hardware configuration. Each stage performs a distinct engineering task and contributes to the successful realization of the final hardware.

- The distinction between synthesis and implementation became particularly clear during the project. Synthesis determines the logical hardware that should exist, whereas implementation determines the physical location and routing of every FPGA resource. Although both stages are automated by Vivado, they solve fundamentally different optimization problems.

- Another important realization was that Verilog is never executed by the FPGA. Once synthesis completes, the RTL is transformed into FPGA primitives such as lookup tables, flip-flops, memories, carry chains, routing resources, and clock networks. Programming the FPGA configures these physical resources rather than loading software instructions into memory.

- Successful implementation also depends on satisfying timing constraints. Functional correctness alone is insufficient; setup and hold timing requirements must be met after placement and routing before the design can operate reliably at the target clock frequency.

- The project further demonstrated that design constraints are an integral part of FPGA development. While the RTL specifies logical behaviour, the XDC constraint file establishes physical connectivity and timing requirements. Both descriptions are required to produce a complete hardware implementation.

- Hardware debugging differs fundamentally from software debugging. Since arbitrary internal signals cannot be inspected directly, designers must intentionally expose important signals through LEDs, integrated logic analysers, oscilloscopes, or other debug interfaces. Signals that do not contribute to observable outputs may be removed entirely during synthesis, making temporary debug ports an essential engineering technique.

- Finally, the project reinforced the importance of incremental hardware bring-up. Rather than debugging an entire processor simultaneously, confidence should be established progressively by validating power, clock generation, FPGA programming, board I/O, simple hardware tests, instruction fetch, and finally complete processor execution. Similarly, verification should proceed hierarchically from individual modules to pipeline stages, complete processor integration, and finally FPGA validation. This methodology substantially reduces debugging complexity and accelerates fault isolation.

14.5 Problems Encountered and Their Resolution

| Problem | Root Cause | Resolution | Engineering Lesson |
| --- | --- | --- | --- |
| Empty implementation | No observable outputs | Added LED debug outputs | Synthesis removes dead logic |
| Bitstream generation failed | Missing XDC constraints | Imported and configured Basys 3 constraints | Constraints are part of the design |
| Instruction memory uninitialized | ROM contained no data | Added initial block with machine code | FPGA memories require initialization |
| Unexpected LED behaviour | Hardware not yet isolated | Performed incremental board bring-up | Debug one layer at a time |
| Hardware Manager disconnect messages | Misleading console logs | Verified device status directly | Validate with device state, not only logs |
| Large RTL integration | Interface and width mismatches | Standardized module interfaces | Hierarchical design simplifies integration |

14.6 RTL Compilation and Simulation

After completing the RTL implementation of all processor modules, the entire processor hierarchy was compiled using the Icarus Verilog compiler. Compilation was performed by invoking the compiler on the processor top-level module together with every subordinate RTL module and the processor testbench.

The compilation command used during development is shown below:

The compilation command performs several important tasks simultaneously. First, Icarus Verilog parses every Verilog source file and checks the syntax of each module. It then resolves the complete module hierarchy by matching every instantiated module with its corresponding source file. During elaboration, the compiler verifies module interfaces, port connections, and signal declarations before producing a unified simulation model of the processor.

iverilog -I rtl -o sim/cpu.out rtl/cpu.v rtl/if/pc.v rtl/if/pc_4_adder.v rtl/if/pc_control.v rtl/if/pc_mux.v rtl/if/instruction_memory.v rtl/if/if_id_pipeline.v rtl/id/register_file.v rtl/id/immediate_generator.v rtl/id/main_control_unit.v rtl/id/hazard_detection_unit.v rtl/id/id_ex_pipeline.v rtl/ex/forwarding_unit.v rtl/ex/operand_selection_muxes.v rtl/ex/alu.v rtl/ex/branch_comparator.v rtl/ex/branch_target_adder.v rtl/ex/ex_mem_pipeline.v rtl/mem/data_memory.v rtl/mem/mem_wb_pipeline.v rtl/wb/writeback_mux.v tb/tb_cpu.v

The -I rtl option specifies the search directory for any included Verilog header files, while the -o sim/cpu.out option instructs the compiler to generate the simulation executable named cpu.out.

It is important to note that cpu.out is not the processor itself and is not an FPGA bitstream. Instead, it is a compiled simulation executable representing the complete RTL design. At this stage the processor exists as an executable software model suitable for functional verification.

Running the Simulation

After successful compilation, the simulation executable was executed using the Verilog runtime simulator.

vvp sim/cpu.out

The processor testbench generated the system clock, applied the reset sequence, executed the simulation, and produced a Value Change Dump (VCD) waveform file containing the transitions of internal processor signals throughout execution.

Waveform Generation and Analysis

The generated waveform database was examined using GTKWave.

gtkwave wave/cpu.vcd

GTKWave was used to observe and debug internal processor signals, including the program counter, fetched instruction, pipeline registers, ALU inputs and outputs, forwarding paths, hazard control signals, memory accesses, and write-back data. Waveform analysis provided cycle-by-cycle visibility into processor operation and formed the primary method of functional verification before FPGA synthesis.